- RADIATION PROTECTION FOR PATIENT AND STAFF IN INTERVENTIONAL PROCEDURE

Ndagire Hadijah 1 ,Dr. Okello Emmy 2, Dr. Nyakoojo. B. Wilson 3 , Dr. Omagino .O. John 4

1. Medical Physicist, Uganda Heart Institute

2. Consultant cardiologist , Uganda Heart Institute

3. Senior Consultant Cardiologist , Uganda Heart Institute

4. Consultant ,Cardiothoracic (surgeon ) Uganda Heart Institute

- Email: hadijahndagire @yahoo.com

- Poster ID: P-S8-3

- Background and Goal of the present work

In cardiac diagnostic and interventional procedures, ionizing radiation is commonly employed. The radiation has a small but definite stochastic chance of causing cancer. Low-dose radiation has been demonstrated to cause an increase in circulating lymphocytes and chromosome abnormalities, both of which are surrogate biomarkers of cancer risk. There is no established threshold value for the long-term cancer risk associated with increasing cumulative exposure.

Uganda Heart Institute has biplane cardiac catheterization equipment Siemens Axiom Artis Zee equipment installed in 2012.

It has a flour mounted frontal C-arm (Plane A) and the ceiling suspended lateral C-arm (Plane B). Deterministic risk of skin damage both to the patient and the operator, risk of eye injury to the operator, risk by radiation exposure to the operator as many procedures are carried out in a year over 150 interventional.

The radiation dose received by cardiologists during interventional procedures and other cardiology procedures varied by more than an order of magnitude for the same type of procedure and for similar patient doses.

This paper analyzed occupational radiation protection for cardiologist, Cath

anethologists, Cath technologists, Cath nurses and other staff in the interventional

suite.

Methodology

The radiation exposure in interventional cardiology was determined by a series of

factors that were partly administered by each procedure and partly dependent on

the conduct of the operator and other personnel. The radiation management by the

operator was the most important modifiable factor in radiation protection when

using biplane cardiac X-ray equipment

Simple methods for reducing or minimizing occupational radiation dose included

minimizing fluoroscopy time and number of acquired images, using available

patient dose reduction technologies, using good imaging-chain geometry,

collimating; avoiding high-scatter areas, using protective shielding, using imaging

equipment whose performance was controlled through a quality assurance

program, and wearing personal dosimeters so you know your dose.

Radiation protection education and training for all interventional cardiology

workers, as well as the availability of proper protective instruments and equipment were essential for effective use of these procedures.

Regular review and investigation of personnel monitoring results, accompanied as

appropriate by changes in how procedures are performed and equipment used, ensured continual improvement in the practice of radiation protection in the interventional suite

Appropriate shielding in cath lab

- Conclusions and Acknowledgements

- For the safety of both patients and staff, limiting radiation exposure during interventional procedures was critical.

- Patient doses and operator exposures were greatly decreased thanks to advances in equipment and the adoption of radiation safety protocols